Operation Guide:

1.Insert micro sd card

2.The indicator will be blue when the power is on.
       Notice: A:The indicator will flash slowly when the battery is low, after 2 minutes.            
                          MD80 will power off automatically.
                     B:The indicator turns green and flash quickly when there is no micro sd card.
                     C:MD 80 will enter standby mode if it is not used for 2 minutes when battery 
                         is not low.

3.Recoding Mode
                   (1)Recoding manually: when the sound trigger switch is off. Press ‘Record Switch ’ to 
                        start recording when power on, meanwhile the green  indicator flashes slowly. 
                        Press “Record Switch” again to stop recrding and the files will be saved as “*.AVI” 
                         format.
                       Notice: A: In standby mode, press “Record switch” once to wake up the system 
                                          while the indicator displays blue color and press “Record switch” again to 
                                          activiate recording mode while the indicator displays green color and 
                                          flashes slowly.
                                      B: The indicator will keep in green and stop recording when the micro sd 
                                           card is full, then stop displaying after 30 minutes. You can not press any 
                                           switch to operate when the indicator keep in green.

(2)Sound trigger recording: when the sound trigger switch is on.
                                      A:Turn on the power, MD 80 will enter recording mode  if the sound 
                                          outside is up to 65 decibel.
                                      B: MD 80 will automatically enter standby mode when the sound outside is 
                                           lower than 65 decibel standing for 2 minutes.
                                       C:In the standby mode, if the sound outside is up to 65 decibel, MD80 will 
                                           enter recording Mode again.

4.After recording, the files will be saced automatically.
          Notice:A:  Turn off the power during recording without stopping , recording  
                             stream will not be saved as files.
                       B: In the case of the long-time video recording , the video will be saved as a 
                             file with 800MB automatically every 30 minutes.

5.The indicator will show red when MD80 is powered by usb or AC adapter, when MD80 is full of charge, the indicator will crush out.

6.The indicator sketch map.

| Indicator | State | Motion |
| --- | --- | --- |
| Blue (Power instruction) | Always bright | Power on |
|  | Crush out | Standby |
|  | Flash slowly | Low battery |
|  | Flash quickly | Copy data |
| Green (Record instruction) | Always bright | Full of card |
|  | Flash slowly | No card |
|  | Flash quickly | Recording |
| Red (Charge instruction) | Always bright | Charge up |
|  | Crush out | Full of charge |

Notice: When charge up the MD80, it can also support record, standby and file transfer, meanwhile  the red indicator would keep on and the indicator is show normally in any other states. When mix up more indicators, the color of the indicator would be changed.